# Deconstruction audit - vLLM analysis

# 20240819_100055.jpg

|  | Space type: circulation corridor Overall condition: Fair comp_01 - flush steel fire-rated door leaf Category: Doors & Portals Materials: carbon steel, mineral core, paint coating Salvage tier: refurbish_reuse Hardware: black lever handle, mortise lock, strike plate, door closer (partially visible) Markings: Graffiti: 'BAGI CASO' in black marker; two silver certification/label plates on edge (partially obscured, likely fire rating or manufacturer ID) Connection: hinged Deconstruction notes: Remove hinges and hardware first. Inspect core for moisture or degradation. Door may require sanding and repainting. Check for lead-based paint if pre-1978. Handle with care to avoid denting. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 312, 6, 825, 968 Hazard and diagnostic flags Verify fire rating label content and core material (possible legacy asbestos-containing core in older buildings). Test for lead-based paint on door surface and hardware due to age and graffiti. Inspect door frame for mastic or sealant residue that may require hazardous waste handling. |
| --- | --- |

# 20240819_100836.jpg

|  | Space type: circulation corridor Overall condition: Fair comp_01 - flush steel door leaf with fire-rated core Category: Doors & Portals Materials: cold-rolled steel, mineral wool core, paint coating Salvage tier: refurbish_reuse Hardware: black lever handle, mortise lock, strike plate, surface-mounted hinges Markings: No visible manufacturer labels or certification badges; small black circular device near top right corner may be a fire alarm or smoke detector sensor. Connection: hinged Deconstruction notes: Remove hinges first, then carefully detach door from frame. Inspect core for moisture or degradation. Core may contain asbestos if pre-1980s; test if uncertain. Clean surface and repaint for reuse. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 152, 78, 345, 825 comp_02 - flush steel door leaf with fire-rated core Category: Doors & Portals Materials: cold-rolled steel, mineral wool core, paint coating Salvage tier: refurbish_reuse Hardware: black lever handle, mortise lock, strike plate, surface-mounted hinges Markings: No visible manufacturer labels or certification badges; small black circular device near top right corner may be a fire alarm or smoke detector sensor. Connection: hinged Deconstruction notes: Remove hinges first, then carefully detach door from frame. Inspect core for moisture or degradation. Core may contain asbestos if pre-1980s; test if uncertain. Clean surface and repaint for reuse. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 415, 198, 745, 578 comp_03 - wire mesh storage basket Category: Finishes & Coverings Materials: galvanized steel wire Salvage tier: direct_reuse Markings: No visible markings or labels. Connection: self-standing Deconstruction notes: Lift out manually; no disassembly required. Clean with wire brush and mild detergent. Suitable for reuse in storage or utility areas. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 333, 525, 475, 698 Hazard and diagnostic flags Core material of steel doors may contain legacy asbestos insulation; verify via sampling if building predates 1980. Black baseboard trim may contain lead-based paint; test if building predates 1978. Mastic or sealant at door frame joints may contain VOCs or hazardous adhesives; inspect and test if necessary. |
| --- | --- |

# 20240824_110946.jpg

|  | Space type: exterior stairwell Overall condition: Fair comp_01 - galvanized steel handrail and guardrail assembly Category: Railings & Guards Materials: galvanized steel Salvage tier: direct_reuse Markings: No visible labels or markings Connection: welded to stair stringers and anchored to concrete base Deconstruction notes: Disassemble by cutting welds at stringer connections; inspect for corrosion at base anchors; remove with care to avoid bending or kinking tubular sections. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 198, 78, 999, 999 comp_02 - galvanized steel stair treads with diamond plate surface Category: Railings & Guards Materials: galvanized steel Salvage tier: direct_reuse Markings: No visible labels or markings Connection: welded to steel stringers Deconstruction notes: Cut welds at stringer connections; inspect for localized corrosion or pitting; treads may be reused as industrial flooring or platform decking. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 312, 238, 935, 985 comp_03 - steel door with intercom and signage Category: Doors & Portals Materials: steel, glass, plastic Salvage tier: refurbish_reuse Hardware: intercom panel, door handle, lock mechanism, strike plate Markings: White paper signage with text; no visible manufacturer markings Connection: hinged, surface-mounted Deconstruction notes: Remove door hinges and disconnect intercom wiring; inspect door leaf for denting or warping; clean and repaint for reuse in industrial or commercial applications. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 278, 0, 525, 258 Hazard and diagnostic flags Inspect galvanized steel for lead-based paint or zinc-rich coatings; test for presence of hazardous mastic or sealants at base anchors. Verify condition of concrete base and embedded anchors for structural integrity prior to removal. |
| --- | --- |

# 20240826_095451.jpg

|  | Space type: interior window opening / room corner Overall condition: Good comp_01 - interior window sill and frame assembly Category: Finishes & Coverings Materials: polyvinyl chloride (PVC) or painted aluminum, glass Salvage tier: refurbish_reuse Markings: No visible manufacturer labels or certification badges; surface shows minor paint chipping and dirt accumulation. Connection: dry-bolted or snap-fit to structural frame Deconstruction notes: Remove by detaching from structural frame using screwdriver or pry tool; inspect for sealant or adhesive residue; clean and repaint for reuse in similar applications. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 142, 550, 999, 999 comp_02 - fixed glass window unit Category: Doors & Portals Materials: tempered or laminated glass, aluminum or PVC frame Salvage tier: direct_reuse Markings: No visible labels; glass appears to be double-glazed with visible spacer bar and desiccant beads. Connection: glazed into frame with gasket or sealant Deconstruction notes: Carefully remove glass unit by releasing gasket or sealant; inspect for cracks or delamination; suitable for reuse in non-load-bearing applications. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 0, 0, 789, 792 Hazard and diagnostic flags Verify presence of lead-based paint on window frame or sill prior to sanding or removal; test sealant for VOCs or asbestos if building predates 1980. Inspect glass for laminated safety film or interlayer; if present, handle with care to avoid shattering during removal. |
| --- | --- |

# 20240826_104603.jpg

|  | Space type: interior window bay / balcony access area Overall condition: Good comp_01 - double-glazed casement window unit (two-leaf configuration) Category: Doors & Portals Materials: PVC-U (polyvinyl chloride), tempered glass, aluminum reinforcement, EPDM seals Salvage tier: direct_reuse Hardware: white plastic tilt-and-turn handle, internal locking mechanism, hinge pins (likely steel) Markings: no visible manufacturer labels or certification badges; window glass has decorative snowflake decals (non-structural) Connection: hinged, surface-mounted to window frame with concealed hinge pins Deconstruction notes: Remove by disengaging locking mechanism, releasing hinges, and carefully lifting out. Check for sealant or mastic at frame-to-wall interface; may require gentle prying with flat bar. Glass panes are likely laminated or double-glazed—handle with care to avoid breakage. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 48, 135, 925, 920 comp_02 - interior window sill (likely PVC or painted wood) Category: Finishes & Coverings Materials: PVC-U or painted MDF/wood composite Salvage tier: refurbish_reuse Markings: no visible markings Connection: adhered or dry-fitted to window frame Deconstruction notes: Remove by gently prying from frame; may be glued with silicone or acrylic sealant. Clean and sand for refinishing if surface is intact. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 48, 820, 925, 950 comp_03 - ceiling soffit with horizontal slats (likely PVC or painted wood) Category: Finishes & Coverings Materials: PVC-U or painted wood Salvage tier: refurbish_reuse Markings: no visible markings Connection: screwed or nailed to ceiling joists Deconstruction notes: Remove by unscrewing or prying slats from backing; check for moisture damage or mold at joints. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 0, 0, 999, 140 Hazard and diagnostic flags Verify presence of legacy sealants or adhesives at window frame-to-wall interface—may contain VOCs or formaldehyde; test if necessary. Inspect window glass for laminated safety glazing (PVB interlayer) or low-emissivity coatings—may affect recycling value. Check for hidden insulation or mastic behind window frame—potential for asbestos or CFC-based foams in older builds (though this unit appears modern). |
| --- | --- |

# 20260205_144908.jpg

|  | Space type: locker room or utility washroom Overall condition: Good comp_01 - stainless steel trough sink with integrated backsplash Category: Casework & Millwork Materials: stainless steel (likely 304 or 316 grade) Salvage tier: direct_reuse Hardware: three chrome-plated wall-mounted faucets with lever handles, flexible grey supply hose Markings: no visible manufacturer labels or certification badges; surface appears clean and unmarked Connection: surface-mounted to tiled wall, likely bolted or welded at back edge Deconstruction notes: Remove by disconnecting supply lines and unscrewing mounting brackets from wall. Ensure no hidden welds or adhesives at junctions. Use care to avoid scratching stainless surface. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 175, 162, 885, 825 comp_02 - large-format ceramic wall tiles Category: Finishes & Coverings Materials: ceramic tile, cementitious thin-set adhesive, grout Salvage tier: refurbish_reuse Markings: no visible manufacturer markings; uniform beige/cream color with subtle veining Connection: thin-set adhesive with grouted joints Deconstruction notes: Tiles can be removed with tile removal tools; may require scoring and prying. Check for moisture damage or delamination. Grout may need cleaning or replacement for reuse. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 17, 95, 885, 575 comp_03 - floor tiles Category: Finishes & Coverings Materials: ceramic or porcelain tile, cementitious adhesive, grout Salvage tier: refurbish_reuse Markings: no visible markings; uniform light beige square tiles Connection: thin-set adhesive with grouted joints Deconstruction notes: Remove using tile removal tools; inspect for cracks or chipping. Grout may need reapplication for reuse. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 0, 575, 725, 1000 comp_04 - chrome-plated wall-mounted faucets Category: Hardware & Fixtures Materials: brass or chrome-plated metal, rubber washers, plastic fittings Salvage tier: refurbish_reuse Hardware: lever handles, flexible supply hose (one visible) Markings: no visible brand or model markings Connection: threaded wall mount with compression fittings Deconstruction notes: Disconnect supply lines and unscrew from wall. Clean and inspect for corrosion or mineral buildup. Reuse with new washers if needed. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 280, 175, 665, 325 comp_05 - white compact utility cabinet Category: Casework & Millwork Materials: particleboard or MDF, melamine laminate, steel frame or hinges Salvage tier: material_recycling Hardware: hinges, drawer slide (partially visible) Markings: no visible labels; white finish with slight wear Connection: surface-mounted or freestanding Deconstruction notes: Disassemble by removing doors and drawers. Separate metal components from wood composites. Check for adhesives or laminates. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 100, 500, 350, 890 comp_06 - green plastic step stool Category: Casework & Millwork Materials: recycled HDPE or polypropylene Salvage tier: direct_reuse Markings: no visible markings Connection: freestanding Deconstruction notes: Clean and inspect for cracks or deformation. Suitable for reuse in utility or storage areas. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 235, 715, 415, 990 Hazard and diagnostic flags Verify presence of lead-based paint or hazardous adhesives on wall tiles or cabinet surfaces prior to removal. Inspect behind sink for legacy insulation or mastic that may contain asbestos or formaldehyde-based binders. Check electrical outlet near sink for proper grounding and moisture protection; verify no exposed wiring. |
| --- | --- |

# 20260504_101951.jpg

|  | Space type: entrance portal Overall condition: Good comp_01 - green-painted steel door leaf with glazed panel Category: Doors & Portals Materials: carbon steel, tempered glass, paint coating Salvage tier: direct_reuse Hardware: chrome-plated D-shaped pull handle, surface-mounted latch, strike plate, door closer (not visible but implied by function) Markings: No visible manufacturer markings or certification badges; text on adjacent panels is informational (contact details, 'BUCALETTERE') Connection: hinged Deconstruction notes: Remove hinges carefully to preserve door leaf integrity. Check for sealant or gasket at frame interface; may require gentle prying. Door appears to be in good condition with no visible warping or damage. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 285, 194, 765, 722 comp_02 - metal grating entrance mat Category: Finishes & Coverings Materials: galvanized steel Salvage tier: direct_reuse Markings: No visible markings Connection: surface-laid Deconstruction notes: Lift out manually; no fasteners. Clean and inspect for corrosion before reuse. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 198, 728, 788, 897 comp_03 - side panel with frosted glass and letterbox Category: Casework & Millwork Materials: aluminum or steel frame, frosted glass, metal letterbox insert Salvage tier: refurbish_reuse Hardware: letterbox slot, label plate with text 'BUCALETTERE' Markings: Text: 'BUCALETTERE', 'Anche per MATERIALE DI VOTO' Connection: surface-mounted Deconstruction notes: Remove by detaching from frame or wall. Clean glass and frame; inspect for paint adhesion or corrosion. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 715, 194, 965, 728 comp_04 - side panel with informational signage Category: Casework & Millwork Materials: aluminum or steel frame, frosted glass, printed vinyl or adhesive label Salvage tier: refurbish_reuse Markings: Text: 'Per le vostre richieste potete contattare i nostri servizi via telefono, via e-mail o consultare il nostro sportello elettronico www.stabio.ch' Connection: surface-mounted Deconstruction notes: Remove by detaching from frame or wall. Peel off adhesive labels if needed; frame may be reused. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 95, 194, 285, 728 Hazard and diagnostic flags Verify presence of sealant or mastic at door frame-to-wall interface; may contain legacy VOCs or asbestos if original construction predates 1980. Inspect door core for potential mineral wool or foam insulation; if present, test for hazardous fibers or chemical degradation before handling. |
| --- | --- |

# 20260504_102000.jpg

|  | Space type: circulation corridor Overall condition: Good comp_01 - stainless steel handrail with curved profile Category: Railings & Guards Materials: stainless steel Salvage tier: direct_reuse Hardware: surface-mounted brackets, welded joints Markings: no visible labels or markings Connection: surface-mounted with bracketed supports Deconstruction notes: Remove by unscrewing or unbolting mounting brackets; inspect for weld integrity and surface corrosion; clean with mild detergent and stainless steel polish for reuse. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 178, 527, 782, 998 comp_02 - frameless glass door with metal edge trim Category: Doors & Portals Materials: tempered glass, aluminum or stainless steel trim Salvage tier: direct_reuse Hardware: door handle (not visible), door closer (not visible), glass-to-frame gasket Markings: no visible labels or markings Connection: edge-mounted with gasketed frame Deconstruction notes: Carefully remove glass panel by releasing gaskets and lifting vertically; inspect for edge chipping or scratches; store flat to prevent stress fractures. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 112, 0, 488, 998 comp_03 - paved stone flooring with metal grating Category: Finishes & Coverings Materials: natural stone pavers, galvanized steel grating Salvage tier: material_recycling Hardware: grating fasteners (not visible) Markings: no visible labels or markings Connection: mortared or bedded in sand, grating secured with clips or welds Deconstruction notes: Remove grating by cutting or prying fasteners; stone pavers can be lifted with pry bar and reused if intact; separate metal and stone for recycling. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 0, 600, 998, 998 comp_04 - white metal display stand with glass top Category: Casework & Millwork Materials: steel frame, tempered glass, powder-coated finish Salvage tier: refurbish_reuse Hardware: adjustable shelf brackets, foot levelers Markings: no visible labels or markings Connection: bolted assembly Deconstruction notes: Disassemble by removing bolts; inspect for coating damage; sand and repaint if needed; glass top can be reused if undamaged. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 732, 425, 998, 745 Hazard and diagnostic flags Verify presence of lead-based paint on metal components prior to sanding or cutting; test for VOCs in adhesives or sealants used in glass gaskets; inspect grating for galvanic corrosion if in contact with dissimilar metals. |
| --- | --- |

# 20260504_103344.jpg

|  | Space type: lecture hall / classroom Overall condition: Good comp_01 - acoustic ceiling tile grid system Category: Finishes & Coverings Materials: concrete, aggregate, cementitious matrix Salvage tier: material_recycling Hardware: suspended grid hangers, metal cross-tees Markings: no visible manufacturer markings; uniform gray finish with geometric voids Connection: suspended grid system with wire hangers Deconstruction notes: Remove tiles carefully to avoid dust; grid system may be reused if structurally sound; test for asbestos if pre-1980s construction Bounding box [ymin, xmin, ymax, xmax] (0-1000): 0, 0, 999, 412 comp_02 - wooden classroom tables (rectangular, pedestal base) Category: Casework & Millwork Materials: laminate-faced particleboard, steel pedestal, powder-coated steel base Salvage tier: refurbish_reuse Hardware: steel pedestal legs, adjustable foot glides, table top edge banding Markings: no visible labels; uniform light wood veneer finish Connection: bolted pedestal to tabletop Deconstruction notes: Disassemble by removing bolts from pedestal; inspect for warping or delamination; clean and refinish for reuse Bounding box [ymin, xmin, ymax, xmax] (0-1000): 0, 580, 999, 900 comp_03 - black office chairs (stackable, metal frame) Category: Casework & Millwork Materials: steel frame, polypropylene shell, fabric upholstery Salvage tier: refurbish_reuse Hardware: casters, gas lift mechanism, adjustable backrest, armrests Markings: no visible branding; standard institutional design Connection: welded frame, bolted components Deconstruction notes: Inspect for broken casters or gas lift; clean upholstery; replace if damaged; stack for transport Bounding box [ymin, xmin, ymax, xmax] (0-1000): 0, 580, 999, 999 comp_04 - parquet flooring (herringbone pattern) Category: Finishes & Coverings Materials: hardwood, varnish finish Salvage tier: direct_reuse Hardware: tongue-and-groove joints, subfloor fasteners Markings: no visible labels; consistent grain and finish Connection: nail-down or glue-down to subfloor Deconstruction notes: Carefully pry up planks using a floor scraper; avoid chipping; sand and refinish for reuse Bounding box [ymin, xmin, ymax, xmax] (0-1000): 0, 580, 999, 900 comp_05 - spherical pendant lights (Bauhaus-style) Category: Lighting & Fixtures Materials: aluminum, glass diffuser, electrical wiring Salvage tier: direct_reuse Hardware: ceiling canopy, suspension cable, socket base Markings: no visible labels; minimalist design Connection: suspended via cable from ceiling grid Deconstruction notes: Disconnect electrical supply; remove canopy and cable; inspect for corrosion or damage Bounding box [ymin, xmin, ymax, xmax] (0-1000): 158, 215, 732, 412 comp_06 - vertical blinds (window coverings) Category: Partitions & Enclosures Materials: PVC or aluminum slats, fabric or vinyl tape, metal headrail Salvage tier: refurbish_reuse Hardware: headrail brackets, cord system, slat connectors Markings: no visible branding; white finish Connection: surface-mounted to window frame Deconstruction notes: Remove from headrail; clean slats; inspect for warping or discoloration; replace cords if frayed Bounding box [ymin, xmin, ymax, xmax] (0-1000): 0, 280, 965, 730 comp_07 - relief sculptures (wall-mounted, plaster or stone) Category: Art & Decorative Elements Materials: plaster, gypsum, concrete Salvage tier: direct_reuse Hardware: wall anchors, mounting brackets Markings: no visible labels; classical figures depicted Connection: surface-mounted with mechanical fasteners Deconstruction notes: Carefully remove with support; inspect for cracks or flaking; clean and restore if needed Bounding box [ymin, xmin, ymax, xmax] (0-1000): 180, 445, 235, 600 comp_08 - lectern or teacher's desk (wooden, elevated platform) Category: Casework & Millwork Materials: solid wood, steel support frame Salvage tier: refurbish_reuse Hardware: drawer pulls, metal legs, adjustable feet Markings: no visible labels; light wood finish Connection: bolted frame to tabletop Deconstruction notes: Disassemble by removing bolts; inspect for wood rot or moisture damage; sand and refinish Bounding box [ymin, xmin, ymax, xmax] (0-1000): 410, 525, 565, 610 Hazard and diagnostic flags Verify presence of asbestos in ceiling tiles (pre-1980s construction common); test if necessary Inspect adhesive or mastic used in flooring installation for VOCs or hazardous compounds Check for lead-based paint on wall-mounted fixtures or sculptures if pre-1978 building Assess electrical wiring in pendant lights for insulation degradation or fire risk |
| --- | --- |

# 20260504_103346.jpg

|  | Space type: interior circulation or open-plan space Overall condition: Good comp_01 - precast concrete coffered ceiling panel system Category: Finishes & Coverings Materials: lightweight aggregate concrete, cementitious matrix Salvage tier: direct_reuse Markings: No visible manufacturer markings or certification badges; surface appears unmarked. Connection: dry-bolted or grouted at joints; likely modular grid system with interlocking or keyed edges Deconstruction notes: Remove panels carefully by lifting from perimeter to avoid cracking; inspect for embedded fasteners or concealed wiring; avoid impact to preserve integrity of voids and surface texture. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 0, 0, 999, 940 comp_02 - spherical pendant light fixtures Category: Lighting & Fixtures Materials: polished metal (likely aluminum or stainless steel), glass or acrylic diffuser Salvage tier: refurbish_reuse Hardware: suspension rods, ceiling canopy, electrical junction box Markings: No visible labels; fixtures appear to be standard commercial design. Connection: suspended via threaded rods or cables from ceiling grid Deconstruction notes: Disconnect electrical supply before removal; inspect for corrosion or damage to suspension hardware; clean and repaint if needed for reuse. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 125, 650, 750, 999 Hazard and diagnostic flags Verify presence of concealed wiring or conduit within ceiling voids before panel removal. Test for presence of legacy fireproofing or sealants in joint areas; potential for asbestos-containing materials if building predates 1980s. Inspect for moisture intrusion or efflorescence in concrete panels, which may indicate long-term water exposure. |
| --- | --- |

# 20260504_105544.jpg

|  | Space type: mechanical room Overall condition: Good comp_01 - Hoval brand wall-mounted boiler unit Category: Mechanical & Plumbing Fixtures Materials: galvanized steel, insulated composite panel, plastic casing Salvage tier: refurbish_reuse Hardware: service access panel, pressure relief valve, gas inlet connection, drain valve Markings: Hoval logo, Hoval Service label with phone number 0848 848 464, equipment ID 13000283, warning triangle symbol Connection: wall-mounted with bolted flange, piped connections via threaded and compression fittings Deconstruction notes: Disconnect gas, water, and electrical supply before removal. Remove mounting bolts and carefully lower unit to avoid damage to casing. Inspect for internal corrosion or heat damage prior to refurbishment. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 142, 158, 792, 632 comp_02 - red expansion tank (likely for heating system) Category: Mechanical & Plumbing Fixtures Materials: steel, polyester coating, rubber diaphragm Salvage tier: refurbish_reuse Hardware: pressure gauge, valve, bracket mounting Markings: No visible labels; standard industrial red finish Connection: bolted to wall bracket, threaded pipe connection Deconstruction notes: Drain and depressurize before removal. Check for internal corrosion or diaphragm degradation. Reuse only if pressure-tested and functional. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 0, 465, 273, 999 comp_03 - gray plastic expansion tank or buffer tank Category: Mechanical & Plumbing Fixtures Materials: polypropylene, steel end caps Salvage tier: material_recycling Hardware: valve, pressure gauge, bracket mounting Markings: No visible labels Connection: bolted to wall bracket, threaded pipe connection Deconstruction notes: Remove from mounting bracket. Separate metal end caps from plastic body for material sorting. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 688, 712, 999, 900 comp_04 - gray PVC or CPVC piping system Category: Plumbing & Piping Systems Materials: PVC, CPVC, brass fittings Salvage tier: material_recycling Hardware: elbows, tees, valves, compression fittings Markings: No visible labels; standard gray coloration Connection: solvent-welded joints, threaded connections Deconstruction notes: Cut pipes at joints to separate fittings. Sort by material type (PVC/CPVC) and metal fittings for recycling. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 62, 105, 999, 735 comp_05 - ceramic tile flooring Category: Finishes & Coverings Materials: ceramic tile, cementitious mortar, epoxy grout Salvage tier: material_recycling Hardware: none Markings: none Connection: adhered to concrete subfloor Deconstruction notes: Remove using tile puller or chisel. Separate tiles from mortar for recycling. Test grout for hazardous additives if unknown. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 192, 835, 999, 999 Hazard and diagnostic flags Verify presence of asbestos in joint compounds or insulation around pipes prior to dismantling. Test for lead-based paint on metal components if building predates 1978. Inspect boiler casing for internal corrosion or heat damage before reuse. Check for mastic or adhesive residues on pipe joints that may require hazardous waste handling. |
| --- | --- |

# 20260504_105610.jpg

|  | Space type: mechanical room or utility closet Overall condition: Good comp_01 - Hoval TOPGAS® CLASSIC (80) condensing boiler Category: Mechanical & Plumbing Equipment Materials: stainless steel, galvanized steel, plastic casing, copper tubing, insulation (mineral wool or polyisocyanurate) Salvage tier: direct_reuse Hardware: pressure relief valve, gas inlet connection, electrical terminal block, drain valve, temperature sensor ports Markings: Manufacturer: Hoval Aktiengesellschaft FL - 9490 Vaduz; Model: TOPGAS® CLASSIC (80); Date: 10/2022; CE Mark: 0085/22; SVGW SSIGE certification; Gas type: ERDGAS H/18 MBAR; Electrical: 230V~/50HZ/10A, IP40D; NOx Class: 6; Serial: 605194700257; Multiple regional gas supply codes (e.g., I2E(R)3P, I2H3B/P); Fire rating: Not applicable; DIN/EN standards: EN 15502 Connection: flanged or threaded gas connections, electrical conduit entry, drain pipe connection, water inlet/outlet ports Deconstruction notes: Disconnect gas, water, and electrical supply lines before removal. Remove mounting brackets. Inspect for corrosion on flue and heat exchanger. Verify integrity of insulation before handling. Use lifting equipment for safe removal due to weight. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 34, 232, 975, 705 Hazard and diagnostic flags Verify presence of legacy insulation materials (e.g., asbestos-containing mineral wool) in boiler casing or flue; test if necessary. Check for residual gas or water in internal piping before dismantling; depressurize and drain system. Inspect for mastic or sealant residues on connections; may require chemical removal if contaminated. |
| --- | --- |

# DSC_0038.JPG

|  | Space type: circulation corridor Overall condition: Fair comp_01 - cast iron column radiator Category: MEP Fixtures & Components Materials: cast iron, enamel paint coating, brass valve Salvage tier: direct_reuse Hardware: thermostatic valve (right end), air vent (left end), bracket mounting hardware Markings: no visible manufacturer markings or certification badges; valve has small circular label with indistinct text Connection: threaded pipe connections, wall-mounted via brackets Deconstruction notes: Remove by disconnecting supply/return pipes using pipe wrench; ensure system is drained and depressurized. Lift radiator carefully to avoid damaging enamel coating. Inspect for internal corrosion via valve access if reuse is intended. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 132, 280, 968, 900 Hazard and diagnostic flags Verify presence of lead-based paint on radiator surface prior to handling or sanding; test for lead if refurbishment is planned. Inspect wall behind radiator for potential asbestos-containing insulation or mastic, especially near baseboard area where discoloration is visible. |
| --- | --- |

# DSC_0758.JPG

|  | Space type: circulation corridor Overall condition: Fair comp_01 - rectangular terracotta floor tile Category: Finishes & Coverings Materials: terracotta clay, mineral aggregate, glaze Salvage tier: direct_reuse Markings: No visible labels or markings; surface shows minor wear and darkened grout lines. Connection: mortared Deconstruction notes: Tiles can be removed intact using a tile removal tool or chisel with care to avoid cracking. Grout joints should be scored first. Use dust suppression. Check for underlying subfloor condition before removal. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 0, 0, 1000, 1000 Hazard and diagnostic flags Verify grout composition for potential asbestos or Portland cement-based mastic; test if necessary before removal. Inspect tile edges for hidden lead-based paint or sealant residues, especially if building predates 1978. |
| --- | --- |

# IMG_2761.HEIC

|  | Space type: interior corridor or office partition zone Overall condition: Good comp_01 - textured white wall finish (likely painted gypsum board or plaster) Category: Finishes & Coverings Materials: gypsum, plaster, acrylic paint Salvage tier: material_recycling Connection: adhered to structural frame Deconstruction notes: Remove drywall or plasterboard carefully to avoid dust; separate metal studs from gypsum core for recycling; inspect for hidden wiring or plumbing before cutting. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 0, 0, 999, 920 comp_02 - wall-mounted electrical outlet (Schuko-type, 230V, 16A) Category: MEP Fittings Materials: polycarbonate, brass, copper Salvage tier: refurbish_reuse Hardware: screwed faceplate, internal terminal screws Markings: standard European Schuko socket marking; no visible brand or certification badge Connection: surface-mounted, screwed to wall Deconstruction notes: Disconnect power supply before removal; inspect for corrosion or damage to internal contacts; clean and test for continuity before reuse. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 248, 830, 345, 865 comp_03 - split-type air conditioning indoor unit (Daikin brand) Category: MEP Fittings Materials: galvanized steel, plastic, copper tubing, refrigerant (R410A or similar) Salvage tier: refurbish_reuse Hardware: mounting brackets, screws, drain pipe connection, power cord Markings: Daikin logo visible; model number not legible; likely includes energy efficiency rating sticker (not visible) Connection: surface-mounted, bolted to wall Deconstruction notes: Must be professionally decommissioned by licensed HVAC technician to recover refrigerant; remove mounting brackets and inspect for wall damage; unit may require cleaning and performance testing for reuse. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 585, 0, 999, 130 comp_04 - dark gray vertical accent strip (possibly metal or vinyl panel) Category: Finishes & Coverings Materials: aluminum, PVC, steel Salvage tier: direct_reuse Connection: adhered or mechanically fastened to wall Deconstruction notes: Carefully remove to preserve surface; check for adhesive residue; may be reusable as decorative panel or partition accent. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 450, 0, 575, 920 comp_05 - black baseboard or skirting board Category: Finishes & Coverings Materials: PVC, MDF, vinyl Salvage tier: refurbish_reuse Hardware: screws or nails Connection: nail/screwed to wall Deconstruction notes: Remove with pry bar; inspect for warping or moisture damage; sand and repaint if necessary for reuse. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 0, 910, 999, 935 comp_06 - dark carpet flooring Category: Finishes & Coverings Materials: nylon, polypropylene, rubber or latex backing Salvage tier: material_recycling Connection: glued or mechanically fastened to subfloor Deconstruction notes: Remove using carpet knife; separate backing from fiber; check for contamination; send to specialized carpet recycling facility. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 0, 920, 999, 999 Hazard and diagnostic flags Verify presence of asbestos in older wall finishes or insulation behind drywall; test for lead-based paint if building predates 1978; inspect electrical wiring for damage or outdated insulation; confirm refrigerant type and pressure in AC unit before dismantling. |
| --- | --- |

# IMG_2765.jpg

|  | Space type: circulation corridor Overall condition: Good comp_01 - flush steel door with transom window Category: Doors & Portals Materials: carbon steel, mineral core, polycarbonate or acrylic glazing Salvage tier: refurbish_reuse Hardware: silver lever handle, mortise lock, strike plate, door closer (not visible but likely present) Markings: No visible manufacturer labels or certification badges; door frame appears to have standard industrial finish. Connection: hinged with concealed hinges (likely surface-mounted or mortised) Deconstruction notes: Remove door leaf by disengaging hinges; inspect core for moisture or degradation. Transom glazing may be secured with silicone or gasket—use caution to avoid breakage. Door frame may be anchored with screws or bolts; check for concealed fasteners. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 142, 225, 450, 710 comp_02 - painted concrete block wall with color bands Category: Finishes & Coverings Materials: concrete masonry units (CMU), acrylic or epoxy paint Salvage tier: material_recycling Markings: No visible labels; color bands (blue, white, green) suggest functional zoning or safety marking. Connection: mortared masonry Deconstruction notes: Demolition requires mechanical breaking; paint may be lead-free based on modern standards, but testing recommended. CMU blocks are recyclable as aggregate. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 575, 0, 999, 999 comp_03 - ceramic tile flooring Category: Finishes & Coverings Materials: ceramic tile, cementitious thinset, concrete substrate Salvage tier: material_recycling Markings: No visible markings; uniform tile pattern suggests standard commercial installation. Connection: adhered with thinset mortar Deconstruction notes: Tiles can be removed with chisel and hammer; however, due to size and condition, likely not suitable for reuse. Recycle as construction aggregate or landfill if contaminated. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 0, 695, 460, 999 comp_04 - steel column or portal frame Category: Structural & Enclosure Materials: structural steel, paint coating Salvage tier: direct_reuse Markings: No visible markings; likely standard structural steel section. Connection: welded or bolted to foundation (not visible) Deconstruction notes: Remove with torch or bolt-cutting; inspect for corrosion. Steel is high-value recyclable; if intact, can be reused in new construction. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 450, 0, 595, 999 Hazard and diagnostic flags Verify paint composition on CMU walls for lead content (especially if pre-1978 building). Inspect door core for moisture or mold; legacy mineral cores may contain asbestos if pre-1980s. Test adhesives and sealants used in door transom and tile installation for VOCs or hazardous content. |
| --- | --- |

# IMG_2768.jpg

|  | Space type: circulation corridor Overall condition: Good comp_01 - painted concrete block wall panels (blue, white, green) Category: Finishes & Coverings Materials: concrete, paint (acrylic or latex) Salvage tier: material_recycling Markings: No visible labels or markings; painted surfaces show no manufacturer badges or certification stickers. Connection: monolithic (cast-in-place or masonry laid with mortar) Deconstruction notes: Walls are structural and not designed for disassembly; removal requires mechanical demolition. Painted surfaces may contain lead-based paint if pre-1978; testing recommended. Avoid dust generation during demolition. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 0, 0, 999, 999 comp_02 - steel structural columns Category: Structural Elements Materials: carbon steel Salvage tier: direct_reuse Markings: No visible markings or tags; surface appears painted or coated, possibly with industrial enamel. Connection: welded or bolted to foundation and beam system (not visible in image) Deconstruction notes: Columns are likely bolted or welded to base plates; require structural assessment before removal. Use magnetic particle inspection if reuse is intended. Remove with crane or lifting equipment. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 365, 0, 550, 999 comp_03 - ceiling tile system (linear metal or composite panels) Category: Finishes & Coverings Materials: galvanized steel, mineral fiber, paint Salvage tier: material_recycling Markings: No visible labels; panels appear uniform with no manufacturer badges. Connection: suspended grid system (not visible), likely clipped or snapped into place Deconstruction notes: Ceiling panels can be removed manually with minimal tools. Check for asbestos in older buildings; if pre-1980, testing required. Recycle metal and fiber components separately. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 545, 0, 999, 290 comp_04 - terrazzo or ceramic tile flooring (herringbone pattern) Category: Finishes & Coverings Materials: ceramic tile, grout, adhesive Salvage tier: material_recycling Markings: No visible labels or manufacturer markings on tiles. Connection: adhered to concrete or mortar bed Deconstruction notes: Tiles are likely bonded with mastic or thin-set; removal requires chipping or grinding. Test adhesive for VOCs or hazardous content. Recycle ceramic as aggregate or raw material. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 250, 550, 999, 999 comp_05 - small fixed window (likely for ventilation or light) Category: Windows & Openings Materials: glass, metal frame (aluminum or steel), paint Salvage tier: refurbish_reuse Hardware: small handle or latch visible on frame Markings: No visible labels or certification badges. Connection: set into concrete block wall, likely with mortar or sealant Deconstruction notes: Window can be carefully removed with pry bar and hammer; check for lead paint on frame. Clean glass and frame; replace sealant if needed. Reuse in low-load applications. Bounding box [ymin, xmin, ymax, xmax] (0-1000): 730, 345, 805, 465 Hazard and diagnostic flags Paint on concrete blocks may contain lead; testing recommended before demolition. Ceiling panels may contain asbestos if building predates 1980; sample for analysis. Flooring adhesive may contain VOCs or hazardous mastic; test prior to removal. Structural steel columns may have coating degradation or corrosion; inspect for pitting or rust before reuse. |
| --- | --- |